# Water Treatment Milwaukee

The Milwaukee Water Works treats Lake Michigan water at the Linnwood Water Treatment Plant on the north side and the Howard Avenue Water Treatment Plant on the south side. The Linnwood intake is  1.25 miles from the shore at a depth of 60 feet. The Texas Avenue intake supplying the Howard Avenue Water Treatment Plant is 2.5 miles from shore, also at a depth of 60 feet.  The lake water passes through a multiple barrier treatment process barriers to protect public health. The barriers destroy and remove illness-causing microorganisms in the lake water.

The primary form of disinfection is ozone gas. In the first stage of water treatment, ozone is bubbled into the water in large contactor tanks. This highly reactive gas destroys illness-causing microorganisms such asCryptosporidium and Giardia, and harmful compounds. Ozone breaks apart compounds that can cause taste and odor. Its use reduces the formation of disinfection byproducts.

Particles in the water are then removed through coagulation, flocculation, settling, and biologically active filtration.Chlorine is added as a secondary disinfectant. Fluoride is added to reduce dental cavities. A phosphorous compound is added to control pipe corrosion to prevent lead that may be present in pipes from leaching into the water. Finally, chloramine disinfection maintains a residual in the distribution system to protect against bacterial contamination. The Wisconsin Department of Natural Resources (DNR) requires water utilities to maintain a detectable level of disinfectant throughout the distribution system to maintain bacteriological protection.

All chemicals that are added are certified food grade, safe for human consumption. The Supervisory Control and Data Acquisition System (SCADA) at both treatment plants provides real-time data from chemical feed systems, including ozone, and all water quality monitoring as well as control of water pumping stations and the distribution system. Pure, fresh water arrives at your taps.

#### Water Treatment Process

Milwaukee Water Works Drinking Water Treatment Process

Proceso de tratamiento del agua del Departamento de Agua de Milwaukee

Proceso de tratamiento del agua del Departamento de Agua de Milwaukee

1 Desinfección  por ozono El gas ozono es burbujeado  a través del agua que entra del lago. El ozono destruye  los microorganismos, giardia y criptosporidio inclusive,  que causan enfermedades; además,  controla el sabor y

olor, y reduce la formación  de los subproductos de la desinfección por cloro.

2 Coagulación  y Floculación  Se añade sulfato de aluminio al agua para neutralizar la carga de las partículas  microscópicas en el agua. En seguida,  se mueve ligeramente  el agua para hacer que las partículas  en suspensión se unan a fin de formar flóculos.

3 Sedimentación  Es el proceso  durante el cual los flóculos  se mueven hacia el fondo para ser eliminados  del agua.

4 Filtración biológicamente  activa El agua es filtrada lentamente  al pasar por 24” de

carbón antracita  y 12” de arena fina para eliminar las partículas  muy pequeñas.

5 Desinfección  por cloro Después  de la filtración,

se añade cloro como un desinfectante secundario. Esto brinda protección  adicional  contra microorganismos potencialmente dañinos.

6 Fluorización Está comprobado  que, al añadir una pequeña  cantidad de fluoruro al agua, se reduce la incidencia  de caries en los dientes.

7 Almacenamiento  de agua clara El agua tratada está almacenada  en tanques  subterráneos y, según sea necesario, se bombea por el sistema  de distribución.

8 Control de corrosión Se añade un compuesto  de fósforo  para ayudar a controlar  la corrosión

de las tuberías;  esto ayuda a evitar que el plomo y el cobre pasen de las tuberías  al agua.

9 Protección  con cloramina La cloramina  es una combinación  de amoniaco  y cloro que sirve como desinfectante del agua y protege contra bacterias en el sistema de distribución.

El Departamento de Agua de Milwaukee es miembro de la Asociación de Agua Potable de los Estados Unidos (AWWA), la Asociación de Agencias la Asociación de Agua Potable de